UART DESIGN

VERIFICATION REPORT

Self-Checking Testbench  |  Code Coverage Analysis  |  Block Diagrams

| [object Object] | [object Object] |
| --- | --- |
| [object Object] | [object Object] |
| [object Object] | [object Object] |
| [object Object] | [object Object] |
| [object Object] | [object Object] |
| [object Object] | [object Object] |
| [object Object] | [object Object] |

# 1. Project Introduction

This report documents the design and verification of a parameterized UART (Universal Asynchronous Receiver/Transmitter) in synthesizable Verilog. UART is a fundamental serial communication protocol enabling point-to-point, full-duplex asynchronous data transfer using only two data lines without a shared clock.

The design under test (DUT) is uart_top, integrating three sub-modules: u_baud (baud generator), u_xmit (transmitter FSM), and u_recv (receiver FSM with 2-flop synchronizer, mid-bit sampling, and framing error detection). The verification goal is maximum functional coverage through a self-checking Verilog testbench exercising all FSM states, transitions, data patterns, reset behaviour, and error conditions.

# 2. Objectives

- Study and understand the UART architecture including baud generator, transmitter FSM, and receiver FSM.

- Implement synthesizable, parameterized UART in Verilog targeting 9600 baud at 50 MHz.

- Develop a self-checking Verilog testbench with automatic PASS/FAIL verification.

- Cover all FSM states (IDLE, START, DATA, STOP) and all transitions in TX and RX.

- Verify boundary data values: 0x00, 0xFF, 0x55, 0xAA and mixed patterns.

- Test error conditions: false start detection, bad stop bit, mid-state reset.

- Achieve ≥ 95% code coverage across statement, branch, toggle, FSM state, and FSM transition metrics.

- Identify and document design issues found during verification.

# 3. Design Architecture

## 3.1 Top-Level Architecture

The uart_top module is a wrapper integrating three cooperating sub-modules. The baud generator produces an oversampled uart_clk from the 50 MHz system clock. Both TX and RX use uart_clk as their operating clock. In loopback mode (used for self-verification) the serial output of the transmitter is fed directly into the receiver input.

Figure 1 — uart_top Top-Level Architecture

## 3.2 Module Hierarchy

| Module | Function | Clock Domain |
| --- | --- | --- |
| uart_top | Top-level wrapper; connects sub-modules | sys clk / uart_clk |
| u_baud | Baud rate generator; divides 50 MHz to uart_clk | 50 MHz sys clk |
| u_xmit | Transmitter FSM; parallel-to-serial conversion | uart_clk |
| u_recv | Receiver FSM; serial-to-parallel with synchronizer | uart_clk |

# 4. Timing Behaviour

## 4.1 Baud Rate Generator

u_baud divides the 50 MHz system clock by counting to clk_div then toggling uart_clk:

clk_div = 50,000,000 / (9600 × 32) = 163  sys_clk counts before toggle

uart_clk period  = 2 × 163 × 20 ns = 6,520 ns  (~153 kHz, 16× oversampling)

1 UART bit       = 16 uart_clk cycles  ≈ 104.2 µs  @ 9600 baud

Figure 2 — Baud Generator Timing & UART Bit Period

## 4.2 Transmitter Frame Timing

| Phase | Duration (uart_clk ticks) | Serial Line |
| --- | --- | --- |
| IDLE | Until xmit_h=1 | High (1) |
| START | 16 | Low (0) |
| DATA | 16 × 8 = 128 | LSB-first data bits |
| STOP | 16 | High (1) |
| Full frame | 160 uart_clk ticks | ≈ 1.042 ms @ 9600 baud |

## 4.3 Receiver Sampling

The receiver samples the start-bit mid-point at ticks==4 (half of the 8-tick half-bit window) for false-start rejection. Data bits are sampled at ticks==15 (end of each 16-tick window), capturing the stable centre of every bit.

# 5. Signal Description

## 5.1 Inputs

| Signal | Width | Description |
| --- | --- | --- |
| clk | 1 | 50 MHz system clock — active on posedge |
| rst_n | 1 | Active-low asynchronous reset |
| xmit_h | 1 | Transmit enable — hold high until uart_clk edge seen |
| xmit_data_h | 8 | Parallel byte to transmit |

## 5.2 Outputs

| Signal | Width | Description |
| --- | --- | --- |
| uart_xmit_h | 1 | Serial UART TX output (idle high, LSB-first data) |
| xmit_doneH | 1 | High when TX is idle or just completed a frame |
| xmit_active | 1 | High while TX is actively sending |
| rec_readyH | 1 | Pulses high when a valid frame has been received |
| rec_busy | 1 | High while RX is actively receiving |
| rec_datah | 8 | Received parallel byte (valid when rec_readyH=1) |
| err | 1 | Framing error — set when stop bit sampled as 0 |

# 6. Working of the Design

## 6.1 Transmitter FSM

Figure 3 — u_xmit Transmitter FSM State Diagram

### Phase 1 — Input (IDLE)

TX waits in IDLE: uart_xmit_h=1, xmit_active=0, xmit_doneH=1. The combinational block continuously loads shift=xmit_data_h. When xmit_h is sampled high on a uart_clk posedge, the FSM transitions to START and xmit_doneH is cleared.

### Phase 2 — Transmission (START → DATA → STOP)

START drives uart_xmit_h=0 for 16 ticks. DATA shifts out shift[0] to uart_xmit_h every tick; on each 16-tick boundary, the shift register right-shifts and bit_cnt increments. After bit_cnt reaches 7, the FSM enters STOP which holds uart_xmit_h=1 for 16 ticks.

### Phase 3 — Completion

On STOP completion xmit_doneH pulses high, xmit_active returns to 0, and the FSM re-enters IDLE ready for the next byte.

## 6.2 Receiver FSM

Figure 4 — u_recv Receiver FSM State Diagram

### Phase 1 — Synchronization

The 2-flop synchronizer (sync → sync2) re-clocks the asynchronous uart_rec input into the uart_clk domain, preventing metastability.

### Phase 2 — Start-Bit Detection & False-Start Rejection

IDLE monitors sync2. On a falling edge (sync2==0) the FSM enters START. At ticks==4 (mid half-bit), sync2 is re-checked. If still 0 the start bit is genuine and the FSM moves to DATAOUT. If sync2 returned to 1 it was a glitch — the FSM returns to IDLE (false-start rejection).

### Phase 3 — Data Capture

DATAOUT samples sync2 at ticks==15 into the shift register with MSB-first packing: shifter<={sync2, shifter[7:1]}. After 8 bits the FSM enters STOP.

### Phase 4 — Stop-Bit Validation

STOP waits 16 ticks. At ticks==15: if sync2==1 (valid stop bit) rec_datah is updated and rec_readyH asserted; if sync2==0 the err flag is set (framing error) and rec_datah is not updated.

# 7. Testbench Architecture

The testbench (uart_tb) directly instantiates the UART DUT and uses three task abstractions for stimulus: uart_transmit, uart_receive, and uart_false_start. It is fully self-checking — every receive comparison prints an automatic PASS or FAIL.

Figure 5 — Testbench Architecture

# 8. Testbench Components

## 8.1 Clock & Reset

50 MHz clock (20 ns period). do_reset holds sys_rst_l low for 4 cycles then releases, returning all FSMs to their IDLE states.

## 8.2 clk_enable Synchronization (Key Fix)

The TX FSM only samples xmitH inside 'else if (tx_enable)' — one sys_clk every 651 cycles. The testbench exposes dut.clk_enable via hierarchical reference and the wait_clk_enable task holds xmitH high until the enable fires, guaranteeing the FSM sees the signal.

wire clk_enable = dut.clk_enable;

task wait_clk_enable;

@(posedge sys_clk);

while (!clk_enable) @(posedge sys_clk);

endtask

## 8.3 uart_transmit Task

Asserts xmitH, waits for the next clk_enable, then drops xmitH and polls xmit_doneH with a 2 M-cycle timeout.

## 8.4 uart_receive Task

Drives a complete UART frame on uart_REC_dataH: start bit (0) for BIT_CLKS = 16×651 = 10,416 sys_clks, each of 8 data bits LSB-first, then stop bit (1). Polls rec_readyH and compares rec_dataH against expected value.

## 8.5 uart_false_start Task

Drives RX line low for only 4×651 = 2,604 sys_clks (less than the 8×651 half-bit check at ticks==4). The receiver sees sync2==1 at its check point and returns to IDLE — verifying the false-start rejection path.

## 8.6 uart_receive_bad_stop Task

Sends a valid 8-bit data frame but drives the stop bit low (0). This exercises the STOP-state else branch where err is set and rec_datah is not updated.

## 8.7 Watchdog Timer

A parallel initial block uses #(64'd500_000_000_000) to enforce a 500-second simulation ceiling, preventing infinite hangs on any missed completion signal.

# 9. Quality Assessment

## 9.1 RTL Design Quality

| Check | Status | Notes |
| --- | --- | --- |
| No latches | PASS | All outputs registered; combinational block fully enumerated |
| Reset completeness | PASS | All registers reset in negedge rst_n branches |
| FSM encoding | PASS | 2-bit binary; all 4 states used; no illegal state reachable |
| Synchronizer | PASS | 2-flop sync (sync, sync2) on async RX input |
| Error detection | PASS | Framing error flag on bad stop bit |
| False-start reject | PASS | Half-bit mid-sample in START state filters glitches |
| Parameters | PASS | WORD_LEN, XTAL_CLK, BAUD via `define macros |

## 9.2 Testbench Quality

| Check | Status | Notes |
| --- | --- | --- |
| Self-checking | PASS | Auto PASS/FAIL for every receive test |
| Clock-synchronised | PASS | xmitH held until clk_enable — no race |
| Timeout guards | PASS | 2M TX, 500K RX, 64-bit watchdog |
| Boundary values | PASS | 0x00, 0xFF, 0x55, 0xAA all tested |
| Error injection | PASS | Bad stop bit and false start exercised |
| Reset coverage | PASS | Reset in IDLE, START, DATA, STOP states |
| Back-to-back | PASS | Consecutive TX and RX frames |

# 10. Test Cases

| TC# | Description | Expected Result | Coverage Target |
| --- | --- | --- | --- |
| TC1 | Reset state check | xmit_doneH=1, rec_readyH=1, all idle | Reset branches |
| TC2 | TX 0x55 | xmit_doneH=1 after full frame | IDLE→START→DATA→STOP→IDLE |
| TC3 | TX 0xAA | xmit_doneH=1 | Alternating bit toggle |
| TC4 | TX 0x00 | xmit_doneH=1 | All-zero data bits |
| TC5 | TX 0xFF | xmit_doneH=1 | All-one data bits |
| TC6 | TX 0xA5 | xmit_doneH=1 | Mixed bit pattern |
| TC7 | IDLE no-op (xmitH=0) | xmit_active=0 stays idle | IDLE→IDLE branch |
| TC8 | RX 0x55 | rec_dataH=0x55 | All RX FSM states |
| TC9 | RX 0xAA | rec_dataH=0xAA | Alternating RX pattern |
| TC10 | RX 0x00 | rec_dataH=0x00 | All-zero RX |
| TC11 | RX 0xFF | rec_dataH=0xFF | All-one RX |
| TC12 | RX 0xA5 | rec_dataH=0xA5 | Mixed RX |
| TC13 | False start | rec_busy=0, rec_readyH=1 | START→IDLE |
| TC14 | Bad stop bit | err set; rec_datah unchanged | STOP F2==0 branch |
| TC15 | Back-to-back TX | Both 0x12, 0x34 sent | STOP→IDLE→START |
| TC16 | Back-to-back RX | Both 0xBE, 0xEF received | Consecutive RX |
| TC17 | Reset in TX START | xmit_active=0, xmit_doneH=1 | Reset in START |
| TC18 | Reset in TX DATA | xmit_active=0, xmit_doneH=1 | Reset in DATA |
| TC19 | Reset in TX STOP | xmit_active=0, xmit_doneH=1 | Reset in STOP |
| TC20 | Reset in RX DATA | rec_busy=0, rec_readyH=1 | Reset in RX DATAOUT |
| TC21 | TX after reset | TX PASS 0xC3 | Post-reset TX |
| TC22 | RX after reset | RX PASS 0x3C | Post-reset RX |
| TC23 | xmitH held multi-cycle | Single frame only | No double-start |

# 11. Simulation Results

## 11.1 Tools

- Simulator: QuestaSim X-2025.06-SP1_Full64

- Coverage: QuestaSim built-in (statement, branch, toggle, FSM state, FSM transition, FEC)

- Waveform: VCD dump via $dumpfile / $dumpvars for GTKWave analysis

## 11.2 Results Summary

| Test Case | Result | Notes |
| --- | --- | --- |
| TC1 Reset state | PASS | All outputs correct after reset |
| TC2 TX 0x55 | PASS |  |
| TC3 TX 0xAA | PASS |  |
| TC4 TX 0x00 | PASS |  |
| TC5 TX 0xFF | PASS |  |
| TC6 TX 0xA5 | PASS |  |
| TC7 IDLE no-op | PASS | FSM remained in IDLE |
| TC8 RX 0x55 | PASS | rec_dataH=0x55 verified |
| TC9 RX 0xAA | PASS |  |
| TC10 RX 0x00 | PASS |  |
| TC11 RX 0xFF | PASS |  |
| TC12 RX 0xA5 | PASS |  |
| TC13 False start | PASS | rec_busy=0, rec_readyH=1 after glitch |
| TC14 Bad stop bit | PASS | err set; rec_datah not overwritten |
| TC15 B2B TX | PASS | 0x12 and 0x34 sent consecutively |
| TC16 B2B RX | PASS | 0xBE and 0xEF received correctly |
| TC17 Reset START | PASS |  |
| TC18 Reset DATA | PASS |  |
| TC19 Reset STOP | PASS |  |
| TC20 Reset RX DATA | PASS |  |
| TC21 TX post-reset | PASS |  |
| TC22 RX post-reset | PASS |  |
| TC23 xmitH held | PASS | Single frame, no double-start |

# 12. Coverage Report

## 12.1 Coverage Summary

| Scope | Total % | Statement % | Branch % | FEC Cond % | Toggle % | FSM State % | FSM Trans % |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TOTAL | 99.00 | 100.00 | 95.00 | 100.00 | 100.00 | 100.00 | 100.00 |
| dut | 100.00 | -- | -- | -- | 100.00 | -- | -- |
| baud | 100.00 | 100.00 | 100.00 | -- | 100.00 | -- | -- |
| tx | 98.82 | 100.00 | 94.11 | 100.00 | 100.00 | 100.00 | 100.00 |
| rx | 99.04 | 100.00 | 95.23 | 100.00 | 100.00 | 100.00 | 100.00 |

## 12.2 Coverage Observations

- All executable statements reached in all three sub-modules. Statement Coverage — 100%:

- Minor gap from tool-generated implicit else branches on fully-enumerated case statements (all 4 states covered; the 2-bit default/unknown states are unreachable by design). Branch Coverage — 95%:

- Every signal bit toggled 0→1 and 1→0, confirming all data patterns (0x00, 0xFF, alternating) were exercised. Toggle Coverage — 100%:

- All 4 states (IDLE, START, DATA, STOP) visited in both TX and RX FSMs. FSM State Coverage — 100%:

- All 12 FSM transitions hit including IDLE→IDLE, START→IDLE (false start), DATA→DATA (mid-frame), and STOP→IDLE. FSM Transition Coverage — 100%:

- All combinational conditions (ticks==15, bit_cnt==7, sync2==0/1) toggled true and false. FEC Condition Coverage — 100%:

## 12.3 Residual Gap Analysis

The 5% branch gap is in tool-generated coverage for combinational always@(*) blocks. QuestaSim creates implicit else branches for case statements even when all states are enumerated. Since both FSMs are 2-bit with all 4 states covered, these implicit branches (for the unreachable 2'b?? encoding) cannot be exercised functionally. This is an expected and acceptable gap — 100% functional coverage is achieved.

# 13. Design Issues Found & Fixed

| # | Issue | Location | Severity | Fix |
| --- | --- | --- | --- | --- |
| 1 | err flag is never cleared — stays high after first framing error | u_recv | Medium | Clear err<=0 in reset branch and at start of IDLE state |
| 2 | rec_busy not deasserted in STOP state — stays high until next IDLE cycle | u_recv STOP | Medium | Add rec_busy<=0 at ticks==15 in STOP alongside rec_readyH<=1 |
| 3 | Testbench: xmitH pulsed 1 sys_clk — FSM never sees it (TX FSM Trans 0%) | Testbench | Critical | Hold xmitH until wait_clk_enable returns |
| 4 | Testbench: bit period calculated as 651×32 instead of 651×16 — all RX fail | Testbench | Critical | BIT_CLKS = 16 × CLK_VALUE = 10,416 sys_clks |
| 5 | Watchdog literal 500_000_000_000 overflows 32-bit — compile error | Testbench | Critical | Use #(64'd500_000_000_000) |

# 14. Conclusion

A fully self-checking Verilog testbench was developed and executed for a parameterized UART running at 9600 baud on a 50 MHz clock. Twenty-three test cases covered all FSM states and transitions, all boundary data patterns, error conditions, back-to-back transfers, and mid-state resets.

- 99% total coverage across all modules

- 100% statement coverage in all three sub-modules

- 100% FSM state and transition coverage in both TX and RX

- 100% toggle coverage confirming all data patterns exercised

- 100% FEC condition coverage

- 95% branch coverage — residual gap is tool-generated unreachable implicit else branches

Three design issues and three testbench bugs were identified and documented. The testbench bugs (clk_enable synchronization, bit-period calculation, 64-bit literal) were fixed to achieve the final coverage numbers shown.

# 15. Future Work

- Parameterize testbench for multiple baud rates (1200, 4800, 9600, 115200) and word lengths (5–9 bits).

- Add constrained-random data generation using SystemVerilog to sweep all 256 byte values automatically.

- Implement formal covergroup / coverpoint constructs for protocol-level functional coverage.

- Add parity support (even/odd/none) to design and verification environment.

- Integrate with a UVM-lite methodology for reusable, scalable verification.

- Target FPGA (Xilinx Artix-7 or Intel Cyclone V) and validate with a physical serial terminal.

- Add SVA assertions for protocol compliance formal verification.

— End of Report —